Note: Only classes A,B,C of IPv4 can be assigned to devices.D and E classes have special purposes.

Question: How does a company get their own network to use.

Answer: IP addresses are assigned to companies or organizations by a non-profit American corporation called the IANA(Internet Assigned Numbers Authority).

Note: The IANA   assigns IPV4 addresses/networks to companies based on their size.

Note: For example,a very large company might receive a class A or class B network(because there are lots of available addresses to use for hosts in each class A and class B network),while a small company might receive a class C network(because there are fewer addresses in each class C network,only 256).

Note:  However,this lead to many wasted IP addresses ,so multiple methods of improving this system have been created.

Explanation of wasted IP addresses

Note: These two router each have three different networks connected to them.However,there is one more network there.That’s the network connecting these two routers.This is known as a ‘point-to-point’ network,meaning that it’s a network connecting two points,in this case R1 and R2.For example this might be a connection between offices in different cities,let’s say San Francisco and New York.Because this is a point-to-ppoint network we don’t need a large address block,so  we use a class C  network.So here is that network with 254 wasted IP addresses and used IP addresses(on the image below).

Example of wasted classes

Note: The total IPv4 address space includes over 4 billion addresses(232 ),and that seemed like a huge number,but now address space exhaustion is a  big problem,there’s not enough addresses.One way to solve this problem is CIDR.

CIDR(Classless inter-domain routing)

CIDR and Subnets

The same point-to-point network from example above

Note: All 1’s in the subnet mask(also known as netmask) indicate that the same bit in the address is the network portion.

Note:  We have 254 usable addresses but we use only  2 of them.

Note: However,CIDR allows us to assign different prefix lengths,so it doesn’t have to be for ex. /24.

Using /25 prefix length

Note: In this case second row is the subnet mask(in dotted decimal version it is 255.255.255.128).

Note: Purple part(purple digit) in subnet mask is also the network portion.So there are 7 bits in the host portion.

Note: When we use /25 prefix length,there is also 126 usable addresses,and we use only 2 of them,so we waste 124 addresses.So that’s better than wasting 252 addresses with /24 prefix length,but it’s still wasteful.

Perfect prefix length for this network (/30 prefix length)

Note: There are 2 usable addresses,because other 2 are network address and broadcast address.We assign 2 usable addresses to Routers and there are no wasted addresses.

Note:  So,instead of 203.0.113.0/24,we will use 203.0.113.0/30,which is a subnet of that larger class C network 203.0.113.0/24.That subnet includes the address range of 203.0.113.0 through 203.0.113.3.

About /30 prefix length

Note: In box with two IP addresses(in binary) are two usable addresses when using /30 prefix length.

About using /31 prefix length

Note: A network with /31 prefix length has only two IP addresses.Normally it would be a problem because it leaves no usable addresses after subtracting the network and broadcast addresses ,but for point-to-point networks like this,a dedicated connection like this between two routers,there is actually no need for a network address or a broadcast address.So we can break the rules in this  case and assign the only two addresses in the network to our routers.

Note: On a cisco router we will get a warning like this ,reminding us to make sure that this is a point-to-point link,but it is a totally valid configuration.

Note: This is even better than using /30 prefix length in previous network,because we’ve saved even more addresses,using only two addresses instead of using 4 in this point-to-point connection.

Note:  People still use /30 prefix length,but /31 prefix length are totally valid and more efficient than /30 prefix length so it is recommended to use /31 prefix length.

About /32 netmask

Note: /32 netmask is never used to configure an actual interface

Note: There are some uses of /32 netmask,for example when we want to create static route not to a network,but just to one specific host,so we can use a /32 mask to specify that exact host.

Chart showing the dotted decimal subnet masks,and their equivalent in CIDR notation

Note: In this chart /25,/26… etc are called CIDR notation,because it was introuduced with CIDR system.

Note: So the use of subnetting is to divide a larger networks into smaller networks,called subnets.

Using /31 instead of /24

Note: So instead of using the whole 203.0.113.0/24 network for the point-to-point connection,we can use a /30 subnet and use only 4 addresses,or even better use a /31 subnet and use only 2 addresses.

Example of subnetting

Note: There are 4 networks connected to R1.There are 45 hosts(including the IP address of R1’s interface) per switch.

Note: We have received the 192.168.1.0/24 network.And we must divide the network in the way it is written in text below the network topology.

Note: So we need 47 addresses per network(because we need also network address and broadcast address also).So we need totally 188 addresses.

Note: After checking the number of some prefix lengths,we find that best mask is /26 mask,because it has 62 usable addresses./26 is the best because /27 doesn’t provide enough space for 47 addresses.

Finding the best netmask(/26 netmask)